Fishycatchy RTOS & C++ Project Work Author Hannes Stalder, Rafael Pieren Version and date Version 1.0, 18.03.2026

Bern University of Applied Sciences Engineering and Computer Science Electrical Engineering and Information Technology

# Summary

Many fishermen face the same issue: during long waiting periods, their attention drifts, causing them to miss the exact moment when a fish bites. This problem becomes even more significant at night, where bite detection is typically limited to simple mechanical indicators such as small bells, which are often unreliable or easy to overlook.

This project presents a solution to these challenges in the form of an electronic buoy acting as a bite indicator. The device detects fish activity in real time and provides visual feedback through integrated lighting. In addition, the system can be configured via a web application.

# System Context

The Fishy Catchy device is a self-contained embedded system built around a custom ESP32 Pico v3 board. The system has two physical inputs and two physical outputs.

Inputs:

- IMU (Inertial Measurement Unit): A 3-axis accelerometer connected via I2C (addresses 0x4C or 0x6C, 400 kHz). It continuously measures acceleration caused by a fish pulling the line. This is the primary detection sensor.

- WiFi (IEEE 802.11): The ESP32 opens a software access point on boot. A phone or laptop connects to configure the device and observe live sensor data.

Outputs:

- LED strip: A 7-LED WS2812 addressable RGB strip (GPIO 25) provides the main visual feedback. It renders different patterns depending on system state.

- Motor: A DC motor (GPIO 19 wake, GPIO 20 enable, GPIO 4 reverse) provides a mechanical response when a fish bite is confirmed.

The ESP32 runs FreeRTOS with tasks distributed across both cores. Core 0 handles the WiFi access point and HTTP server. Core 1 handles all real-time tasks: sensor sampling, detection processing, and LED rendering. Two mutex-protected shared stores (ConfigStore and SystemStateStore) allow safe communication between cores.

# Requirements

| ID | Requirement | State |  |  |
| --- | --- | --- | --- | --- |
| R1 | The system shall detect a fish bite using IMU acceleration data with a configurable threshold. | Verified by shaking device; Single-spike and cumulative algorithms trigger; thresholds adjustable via UI. |  |  |
| R2 | The system shall process sensor data at a fixed periodic rate (≤ 100 ms cycle time). | Sensor task runs at 2 ms (500 Hz), confirmed via log timestamps. |  |  |
| R3 | The system shall indicate a detected fish bite via a visible LED pattern within ≤ 200 ms. | LED task polls every 40 ms; chase pattern activates within one frame of catch. |  |  |
| R4 | The system shall operate reliably in low-light and night conditions. | LED feedback visible in complete darkness; tested in darkened room. |  |  |
| R5 | The system shall provide at least 3 distinct LED states (idle, WiFi active, fish caught). | Idle (green breath) and Caught (orange chase) implemented; WiFi-active state missing. |  |  |
| R6 | The system shall start a WiFi Access Point automatically on boot. | WifiTask starts from app_main; "FishyCatchy" SSID visible immediately. |  |  |
| R7 | The system shall disable WiFi automatically if no client connects within a configurable timeout (e.g., ≤ 2 minutes). | 120-second idle timeout hardcoded; shuts down if no HTTP activity detected. |  |  |
| R8 | The system shall keep the WiFi AP active as long as a client is connected. | HTTP requests reset activity timer via MarkActivity; verified by holding config page open. |  |  |
| R9 | The system shall allow the user to configure system parameters (e.g., thresholds, LED behavior) via a web application. | Web UI at 192.168.4.1 allows adjustment of algorithms, thresholds, and brightness. |  |  |
| R10 | The system shall store user settings in non-volatile memory. | ConfigStore persists AppConfig struct in NVS; settings survive power cycles. |  |  |
| R11 | The system shall apply updated configuration only after a system reboot. | Config is saved/read on boot, but tasks also hot-reload on version change (immediate effect). |  |  |
| R12 | The system shall provide real-time telemetry data (sensor values, system status) via WiFi. | /sensor endpoint returns JSON (accel, algorithm, flags); Web UI polls every 300 ms. |  |  |
| R13 | The system shall ensure thread-safe access to shared system state using FreeRTOS synchronization mechanisms (mutex). | SharedConfigStore and SharedSystemState use semaphores for all read/write operations. |  |  |
| R14 | The system shall run communication and real-time tasks on separate cores. | WiFi on Core 0; Sensor, Processor, and LED tasks pinned to Core 1. |  |  |
| R15 | The system shall handle concurrent read/write access to shared memory without data corruption. | All access goes through mutex-protected GetCopy and Update functions. |  |  |
| R16 | The system shall operate continuously for at least 10 minutes without memory leaks or crashes. | Ran for >30 mins during integration with no crashes or observable memory growth. |  |  |
| R17 | The system shall monitor stack and heap usage and remain within allocated limits. | open |  |  |
| R18 | The system shall consume minimal power by disabling unused components (e.g., WiFi when inactive). | WiFi shuts down after 120s; motor/LEDs active only as needed or configured. |  |  |

# Design

## Data Flow Diagram

Level 0 (context diagram):

The Fishy Catchy system has two external entities: the IMU sensor providing raw acceleration data, and the user interacting via WiFi. The system processes sensor input and produces LED output and a WiFi configuration/telemetry interface.

Figure 1: Context Diagram

Level 1 (core split):

The system is split across two processor cores. Core 1 owns the real-time path: it reads from the IMU, runs detection, and drives the LED. Core 0 owns the WiFi path: it serves the HTTP configuration UI and the live sensor endpoint. The two cores exchange data through two shared memory regions: SystemState (written by Core 1, read by Core 0 for telemetry) and Config (written by Core 0 via the web UI save, read by Core 1 for detection parameters). Both regions are mutex-protected to prevent data races.

Figure 2: DFD Level 1

Level 2 (task detail):

Within Core 1, SensorTask reads the accelerometer at 500 Hz over I2C and pushes SensorSample structs into a FreeRTOS queue (depth 48). ProcessorTask blocks on this queue, applies the configured detection algorithm, and writes results to SystemStateStore. LedTask reads SystemStateStore at 25 fps and renders the appropriate LED pattern.

Figure 3: DFD Level 2

## Class Structure Diagram

The diagram shows five active participants: the entry point (MainApp), two shared data stores (ConfigStore, SystemStateStore), a FreeRTOS queue (SensorQueue), and four FreeRTOS tasks. MainApp initializes the stores and starts all four tasks. The data flow follows a pipeline: SensorTask produces samples into the queue, ProcessorTask consumes them and writes detection results to SystemStateStore. LedTask reads SystemStateStore to decide which LED pattern to show. WifiTask is the only component with write access to ConfigStore, representing the user-facing configuration path. All other tasks only read from ConfigStore.

Figure 4: Class Strucutre Diagram

## Detection State Machine

After startup the system transitions from IDLE to MONITORING as soon as the sensor task delivers the first valid sample. In MONITORING, every incoming SensorSample is evaluated against the configured threshold. Two algorithms are available: Single Spike checks whether the current absolute sum of the three acceleration axes exceeds a threshold in one sample. Cumulative Average computes a running average over a configurable time window and compares that average to a threshold. If the threshold is not exceeded the system continues collecting. When a detection fires, the system registers the catch timestamp in SystemStateStore and enters a cooldown period (default 2500 ms, configurable). During cooldown, new detections are suppressed to prevent a single event from triggering multiple catches. After cooldown the latch is cleared and the system returns to MONITORING. The LED chase pattern remains active for 5 seconds (kCatchVisualMs) regardless of cooldown duration, giving a clear visual indication even for short cooldown settings.

Figure 5: Detection State Machine

## Web Interface

The web interface is a single-page HTML application embedded as a string literal inside wifi_task.cpp and served directly from the ESP32 at 192.168.4.1. No external files or network access are required.

The page has two tabs. The Settings tab presents all configurable parameters as form inputs: a dropdown to select the detection algorithm (Single Peak or Cumulative Avg), two range sliders for the respective detection thresholds with live numeric readouts, number inputs for the cumulative window time and catch cooldown duration, and a number input for LED brightness. On load the page fetches current values from the /config endpoint (GET, returns JSON) and populates all fields. Pressing Save and Turn Off WiFi submits the form as a URL-encoded POST to /save. The server validates and writes the new config to NVS, signals a shutdown, and the WiFi access point stops. The browser shows a confirmation message.

The Live Sensor tab shows real-time data without any user interaction. A JavaScript setInterval calls the /sensor endpoint (GET, returns JSON) every 300 ms and updates six displayed values: the three raw acceleration axes in g, a valid flag, the current absolute axis sum, the cumulative average, the current detection status, and which algorithm is active. This tab is primarily useful for calibrating thresholds: the user can observe the noise floor and peak values while physically moving the device to determine appropriate threshold settings.

INSERT WEB GUI SCREENSHOT

# Runtime Statistics

| Task | Priority | Stack Allocated | Stack Usage in % (high water mark) | (optional) Average CPU usage in % |
| --- | --- | --- | --- | --- |

| Time | Heap Usage in B |
| --- | --- |
| ~ 10s runtime |  |
| ~ 1min runtim |  |
| ~ 10min runtime |  |

# File Structure

## File Tree

main/

├── CMakeLists.txt

├── idf_component.yml

├── app_types.hpp

├── main.cpp

├── config_store.hpp

├── config_store.cpp

├── system_state.hpp

├── system_state.cpp

├── sensor_task.hpp

├── sensor_task.cpp

├── processor_task.hpp

├── processor_task.cpp

├── led_task.hpp

├── led_task.cpp

├── wifi_task.hpp

└── wifi_task.cpp

## File Descriptions

app_types.hpp

Shared type definitions: board pin constants, AppConfig struct, SensorSample struct, SystemState struct, enums for DetectionAlgorithm and LedPattern.

CMakeLists.txt

ESP-IDF component build configuration. Registers all source files and sets C++17 standard.

idf_component.yml

Component manifest declaring managed dependencies on espressif/cjson and espressif/led_strip.

main.cpp

Application entry point. Initializes NVS, ConfigStore, SystemStateStore, and the sensor queue, then starts all four FreeRTOS tasks.

config_store.hpp

Interface for the NVS-backed config store: init, get a thread-safe copy, update and persist.

config_store.cpp

Implementation: default values, input sanitization, load from NVS on boot, save to NVS on user update.

system_state.hpp

Interface for the shared runtime state store: all update and read functions.

system_state.cpp

Implementation: mutex-protected state struct with per-subsystem update functions to minimize lock contention.

sensor_task.hpp

Interface for SensorTask: start function.

sensor_task.cpp

Reads the accelerometer at 500 Hz over I2C, pushes SensorSamples into the queue, and drives the motor GPIO on catch.

processor_task.hpp

Interface for ProcessorTask: start function and window size constant.

processor_task.cpp

Consumes samples from the queue, runs single-spike or cumulative-average detection, and registers catch events in SystemStateStore.

led_task.hpp

Interface for LedTask: start function and LED count constant.

led_task.cpp

Renders LED patterns at 25 fps on Core 1. Green breath during idle, orange chase for 5 seconds after a catch.

wifi_task.hpp

Interface for WifiTask: start function.

wifi_task.cpp

Runs the WiFi AP and HTTP server on Core 0. Serves the config UI, /config and /sensor JSON endpoints, and /save. Shuts down after 120 seconds idle or immediately after the user saves settings.

# User Manual

Step 1: Power on the device.

Connect a power source to the ESP32 board. The LED strip will start a slow green breathing animation within a few seconds, indicating the system is running and monitoring for bites.

Step 2: Connect to the configuration WiFi.

On your phone or laptop, open the WiFi settings and connect to the network named FishyCatchy. The password is fishycatchy. The network is only available for 2 minutes after power-on if no one connects. If you miss the window, power-cycle the device to restart the access point.

Step 3: Open the configuration page.

Open a web browser and go to 192.168.4.1. The Fishy Catchy configuration page will load. It has two tabs: Settings and Live Sensor.

Step 4: Adjust detection settings.

In the Settings tab you can change the following parameters:

- Algorithm: choose Single Peak to trigger on a single strong jerk, or Cumulative Avg to trigger on sustained vibration over a time window.

- Single Peak (g sum): the acceleration threshold for the Single Peak algorithm. Higher values require a strongerc jerk.

- Cumulative Threshold: the average acceleration threshold for the Cumulative Avg algorithm.

- Window Time (ms): how many milliseconds of samples are averaged for the Cumulative Avg algorithm.

- Catch Cooldown (ms): how long after a detected bite before a new detection is possible. Prevents a single event from counting multiple times.

- LED Brightness: brightness of the LED strip from 0 (off) to 255 (maximum).

Use the Live Sensor tab to see real-time accelerometer values and the current detection status. This is useful for setting thresholds: watch the Abs Sum value while moving the fishing rod and set the Single Peak threshold just above the noise level.

Step 5: Save settings.

Press Save and Turn Off WiFi. The settings are written to non-volatile memory and the WiFi access point shuts down. The device is now ready to use.

Step 6: In use.

Attach the device to your fishing rod or line. The green breathing LED indicates the system is monitoring. When a bite is detected, the LEDs switch to a fast orange chase pattern for 5 seconds and the motor activates briefly. After the visual feedback ends, the system automatically returns to monitoring mode.

To change settings again, power-cycle the device and reconnect within 2 minutes.

# Version control

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1 | 11.03.2026 | First attempts with hardware | Rafael Pieren |
| 0.2 | 13.03.2026 | Hardware tested with independent scripts | Rafael Pieren |
| 1.0 | 18.03.2026 | First working version | Hannes Stalder |